CODE – Signal processing

import pandas as pd

import matplotlib.pyplot as plt

import numpy as np

from scipy.signal import butter, filtfilt, welch

from collections import Counter

# Load dataset

data = pd.read_csv("eeg_data.csv")

print(data.columns)

signal = data["Signal_uV"].values

# Sampling frequency

fs = 256

# -------- BANDPASS FILTER --------

low = 0.5

high = 45

b, a = butter(4, [low/(fs/2), high/(fs/2)], btype='band')

signal = filtfilt(b, a, signal)

# -------- PSD OF ENTIRE SIGNAL --------

freqs_full, psd_full = welch(signal, fs)

plt.figure()

plt.semilogy(freqs_full, psd_full)

plt.xlabel("Frequency (Hz)")

plt.ylabel("Power")

plt.title("Power Spectral Density of EEG")

plt.xlim(0,50)

plt.show()

# -------- SLIDING WINDOW ANALYSIS --------

window_size = 2 * fs

step_size = fs

states = []

times = []

true_labels = []

for start in range(0, len(signal) - window_size, step_size):

segment = signal[start:start + window_size]

freqs, psd = welch(segment, fs)

delta = np.mean(psd[(freqs >= 0.5) & (freqs < 4)])

theta = np.mean(psd[(freqs >= 4) & (freqs < 8)])

alpha = np.mean(psd[(freqs >= 8) & (freqs < 13)])

beta  = np.mean(psd[(freqs >= 13) & (freqs < 30)])

gamma = np.mean(psd[(freqs >= 30) & (freqs < 50)])

total = delta + theta + alpha + beta + gamma

bands = {

"Delta": delta/total,

"Theta": theta/total,

"Alpha": alpha/total,

"Beta": beta/total,

"Gamma": gamma/total

}

dominant = max(bands, key=bands.get)

states.append(dominant)

times.append(start/fs)

# Correct validation label (majority in window)

segment_labels = data["WaveType"].iloc[start:start + window_size]

true_label = segment_labels.mode()[0]

true_labels.append(true_label)

# -------- BRAIN STATE TIMELINE --------

plt.figure()

plt.plot(times, states, '.')

plt.xlabel("Time (seconds)")

plt.ylabel("Dominant Band")

plt.title("Brain State Over Time")

plt.show()

# -------- OVERALL DOMINANT BAND --------

state_count = Counter(states)

dominant_band = state_count.most_common(1)[0][0]

print("Dominant Band:", dominant_band)

# -------- BRAIN STATE CLASSIFICATION --------

if dominant_band == "Delta":

state = "Sleep"

elif dominant_band == "Theta":

state = "Drowsy"

elif dominant_band == "Alpha":

state = "Relaxed"

elif dominant_band == "Beta":

state = "Thinking"

else:

state = "Intense activity"

print("Brain State:", state)

# -------- VALIDATION --------

correct = sum(p == t for p, t in zip(states, true_labels))

accuracy = correct / len(states)

print("Model Accuracy:", accuracy)

# -------- 3 PANEL VISUALIZATION --------

fig, ax = plt.subplots(3, 1, figsize=(10,8))

# Panel 1 — EEG waveform

ax[0].plot(signal[:5*fs])

ax[0].set_title("EEG Signal (First 5 seconds)")

ax[0].set_xlabel("Samples")

ax[0].set_ylabel("Amplitude (uV)")

# Panel 2 — PSD

ax[1].semilogy(freqs_full, psd_full)

ax[1].set_xlim(0,50)

ax[1].set_title("Power Spectral Density")

ax[1].set_xlabel("Frequency (Hz)")

ax[1].set_ylabel("Power")

# Panel 3 — Brain state timeline

ax[2].plot(times, states, '.')

ax[2].set_title("Brain State Over Time")

ax[2].set_xlabel("Time (seconds)")

ax[2].set_ylabel("Dominant Band")

plt.tight_layout()

plt.show()